# User Requirements Document — Wilderness Weather Station

Table of Contents

1. Introduction	1

1.1 Purpose	1

1.2 Scope	2

1.3 Definitions	2

1.4 Source	2

2. Background	2

2.1 The case study	2

2.2 Why the system exists	2

2.3 Existing situation	3

2.4 Classification basis	3

3. Problem Statement	3

4. Objectives	4

Success criteria	4

5. Stakeholders	4

6. Functional Requirements (UR-01 … UR-16)	5

Grouping	7

7. Non-Functional Requirements (UR-17, UR-18)	7

Why these are non-functional	7

Measurable form for the SRS	8

Case study source: Ian Sommerville, Software Engineering, 9th edition (Ch. 1 SS1.3.2 pp. 19-21; Ch. 4 SS4.1 pp. 84-89; Ch. 7 SS7.1 pp. 188-198).

# 1. Introduction

## 1.1 Purpose

This User Requirements Document (URD) states, in the language of the user, what the Wilderness Weather Station system must do. It is the input to the System Requirements Specification (../../SRS/srs.md), where each user requirement is expanded into verifiable system requirements.

## 1.2 Scope

The system covered by this document has three parts:

- the weather station deployed in a remote area, with its instruments, power system and satellite link;

- the data management and archiving system, a central server that collects, processes and stores data from every station;

- the station maintenance system, used to monitor station health and control station parameters remotely.

## 1.3 Definitions

| Term | Meaning |
| --- | --- |
| Station | One self-contained wilderness weather station unit |
| UR | User Requirement, numbered UR-01 … UR-18 |
| Instrument | A sensor measuring temperature, pressure, sunshine, rainfall, wind speed or wind direction |
| Failover | Automatic switch from a failed primary instrument to a backup |
| Dynamic reconfiguration | Replacing a software component while the system keeps running |

## 1.4 Source

All requirements in this document are traced to Ian Sommerville, Software Engineering, 9th edition — Chapter 1 (§1.3.2, pp. 19–21), Chapter 4 (§4.1, pp. 84–89) and Chapter 7 (§7.1, pp. 188–198). Full citation list: ../../references/references.md.

# 2. Background

## 2.1 The case study

The wilderness weather station is the running case study introduced in Sommerville, Software Engineering, 9th ed., Ch. 1, §1.3.2 (pp. 19–21) and carried into the object-oriented design chapter, Ch. 7, §7.1 (pp. 188–198).

Paraphrasing the source (Ch. 1, p. 19, ll. 6–17): stations of this kind gather weather information in remote areas that have no local infrastructure. They must be reliable and run for long periods without anyone attending them, they manage their own power by monitoring batteries and a generator, and they may shut the wind generator down in high winds so it is not damaged.

## 2.2 Why the system exists

Meteorological services need observations from places where no staffed station can be maintained: mountain ranges, deserts, ice fields. A network of unattended stations feeding a central archive gives that coverage, provided each station can survive on its own between visits.

## 2.3 Existing situation

- Weather data from remote regions is sparse or absent.

- Where instruments do exist, readings must be collected manually on site visits.

- There is no single archive that consolidates data from every remote location.

- Instrument or power faults are only discovered on the next visit, so months of data can be lost.

## 2.4 Classification basis

Requirements are classified using Sommerville, Ch. 4, §4.1:

- Functional requirements (p. 85, ll. 3–6) describe the services the system provides, how it reacts to inputs, and how it behaves in particular situations.

- Non-functional requirements (p. 87, ll. 1–4) are constraints on those services — timing, process, standards and similar.

# 3. Problem Statement

Weather observations are needed from areas that have no local infrastructure — no mains power, no wired network, no staff (Sommerville, 9th ed., Ch. 1, p. 19, l. 6). Conventional instrumentation cannot be used there, because it assumes someone is present to power it, read it and repair it.

The problem the system must solve therefore has four strands:

- Autonomy. The station has to run unattended for long periods and cannot depend on human intervention to keep working (Ch. 1, p. 19, l. 12).

- Self-sufficiency in power. With no grid connection, the station must generate, store and ration its own power, and protect its generator in dangerous conditions (Ch. 1, p. 19, ll. 14–17).

- Communication without a network. The only route to the outside world is a satellite link, which is intermittent and expensive, so data must be processed and summarised locally before it is sent (Ch. 1, p. 20, ll. 22–23).

- Maintenance at a distance. Faults must be detected by the station itself, reported to a maintenance system, and where possible corrected remotely — by failing over to a backup instrument or by downloading a replacement software component — because a site visit may be months away (Ch. 7, pp. 195, 197).

Without a system that addresses all four, remote weather data stays unavailable or arrives too late and too incomplete to be useful for forecasting.

# 4. Objectives

| # | Objective | Supporting requirements |
| --- | --- | --- |
| O-1 | Collect accurate periodic measurements of temperature, pressure, sunshine, rainfall, wind speed and wind direction at a remote site | UR-01 |
| O-2 | Reduce raw readings locally into validated summaries so that satellite bandwidth is used sparingly | UR-02, UR-03 |
| O-3 | Consolidate the output of every station into one processed, analysable, permanently archived data set | UR-04, UR-05, UR-06 |
| O-4 | Let operators see the health of every station without travelling to it | UR-07, UR-08, UR-10, UR-11 |
| O-5 | Keep the station powered and protect its power hardware without human help | UR-09, UR-12, UR-13 |
| O-6 | Allow the station to be administered and updated remotely, without a shutdown | UR-14, UR-15 |
| O-7 | Keep collecting data through partial hardware failure | UR-16, UR-17 |
| O-8 | Operate indefinitely in a location with no infrastructure and no staff | UR-18 |

## Success criteria

- A station continues to record and transmit data across a full deployment period with no site visit.

- A failed instrument is detected, reported and worked around automatically.

- A software defect can be corrected from the central site over the satellite link.

- No observation is lost because the archive was unavailable.

# 5. Stakeholders

| Stakeholder | Interest in the system | Requirements they drive |
| --- | --- | --- |
| Meteorologists / weather forecasters | Need timely, trustworthy observations from regions with no coverage, and a long historical record to analyse | UR-01, UR-05, UR-06 |
| Data analysts / climate researchers | Query the archive for long-run trends; care about completeness and consistency of the record | UR-04, UR-05, UR-06 |
| Station maintenance engineers | Must know which station is failing and why, and want to fix what they can from the office | UR-07, UR-08, UR-10, UR-11, UR-14, UR-15 |
| System administrators / operators | Control station parameters, schedule updates, manage the central server | UR-14, UR-15 |
| Software development team | Build the embedded, server and maintenance software; need requirements that can be verified | All |
| Field service technicians | Perform the rare physical visit; want faults diagnosed before they travel | UR-11, UR-16 |
| Funding body / meteorological agency | Pays for the network; wants maximum uptime per station and minimum field visits | UR-17, UR-18 |
| Instrument and hardware suppliers | Supply sensors, batteries, wind generator and satellite transceiver against a fixed specification | UR-09, UR-12, UR-13 |

# 6. Functional Requirements (UR-01 … UR-16)

Definition applied — Sommerville, 9th ed., Ch. 4, p. 85, ll. 3–6: functional requirements state the services the system should provide, how it should react to particular inputs, and how it should behave in particular situations.

| UR ID | Source (page & line) | Source content, paraphrased | User statement |
| --- | --- | --- | --- |
| UR-01 | Ch. 1, p. 20, l. 10 | Stations gather readings from a set of instruments measuring temperature, pressure, sunshine, rainfall, wind speed and wind direction | “The system must collect temperature, humidity, wind speed/direction, rainfall and other sensor measurements periodically.” |
| UR-02 | Ch. 1, p. 20, l. 22 | The station software is responsible for collecting the weather data and performing initial processing on it | “The system must validate, filter and summarise raw sensor data before sending it out.” |
| UR-03 | Ch. 1, p. 20, l. 23 | …and for transmitting that data to the data management system | “The station must transmit collected weather data to the central Data Management and Archiving System.” |
| UR-04 | Ch. 1, p. 20, l. 25 | The central system gathers data from every wilderness weather station | “The central server system must collect and organise incoming data sent from multiple weather stations.” |
| UR-05 | Ch. 1, p. 20, l. 26 | It performs data processing and analysis | “The system must process and analyse weather data to support monitoring and forecasting.” |
| UR-06 | Ch. 1, p. 20, l. 27 | …and archives the data | “The system must archive historical weather data for long-term retrieval and analysis.” |
| UR-07 | Ch. 1, p. 20, l. 30 | The maintenance system uses satellite communication with all stations to monitor their health | “The system must remotely monitor overall station health status via satellite communications.” |
| UR-08 | Ch. 7, p. 191, l. 14 | The system monitors the condition of all instruments and reports problems to the maintenance system | “The station software must continuously test instruments and flag operational anomalies or failures.” |
| UR-09 | Ch. 1, p. 19, l. 15 | The station manages its own power, monitoring its batteries and generator | “The software must monitor battery charge, generator output and report power issues.” |
| UR-10 | Ch. 7, p. 192, l. 8 | The communication system monitors transmission hardware and reports link failures | “The system must monitor satellite communication hardware and log transmission failures.” |
| UR-11 | Ch. 1, p. 20, l. 31 | …and provides reports of problems | “The system must detect critical hardware/software faults and issue diagnostic fault reports to maintenance.” |
| UR-12 | Ch. 1, p. 19, l. 14 | Power has to be managed in situations where the available supply is restricted | “The system must manage power consumption and recharge batteries whenever conditions permit.” |
| UR-13 | Ch. 1, p. 19, l. 17 | The system may shut the wind generator down in high winds so the generator is not damaged | “The system must shut down the generator in hazardous conditions like extreme winds to protect hardware.” |
| UR-14 | Ch. 1, p. 20, l. 29 | The station maintenance system communicates by satellite in order to control station parameters | “The system must support remote administrative login to control station state and perform maintenance.” |
| UR-15 | Ch. 7, p. 197, l. 9 | Dynamic software reconfiguration lets components be updated remotely without shutting the system down | “The system software must support dynamic component replacement/updates remotely with minimal disruption.” |
| UR-16 | Ch. 7, p. 195, l. 18 | If an instrument fails, the system switches to a backup instrument where one is available | “The system must automatically failover to a backup sensor whenever a primary instrument fails.” |

## Grouping

- Data acquisition: UR-01, UR-02, UR-03

- Central data services: UR-04, UR-05, UR-06

- Monitoring and fault reporting: UR-07, UR-08, UR-10, UR-11

- Power management: UR-09, UR-12, UR-13

- Remote administration: UR-14, UR-15

- Fault tolerance: UR-16

# 7. Non-Functional Requirements (UR-17, UR-18)

Definition applied — Sommerville, 9th ed., Ch. 4, p. 87, ll. 1–4: non-functional requirements are constraints on the services or functions the system offers — timing constraints, constraints on the development process, standards, and so on.

| UR ID | Type | Source (page & line) | Source content, paraphrased | User statement |
| --- | --- | --- | --- | --- |
| UR-17 | Reliability / availability | Ch. 1, p. 19, l. 12 | The system has to be reliable and operate without human intervention for long periods | “The overall system must maintain high availability, tolerate partial failures and minimise downtime.” |
| UR-18 | Environmental / operational | Ch. 1, p. 19, l. 6 | These stations collect weather information in remote areas that have no local infrastructure | “The station must operate completely autonomously in remote locations where physical access is difficult.” |

## Why these are non-functional

Neither requirement names a service the system performs. Each constrains how well and under what conditions every service in §6 must be delivered:

- UR-17 constrains all functional requirements with an availability and fault-tolerance target. It is the reason UR-16 (failover) and UR-11 (fault reporting) exist as functions.

- UR-18 constrains the operating environment: no mains power, no wired network, no operator on site. It is the reason UR-09, UR-12 and UR-13 (power management) and UR-03 (satellite transmission) exist as functions.

## Measurable form for the SRS

| UR | Suggested verifiable criterion |
| --- | --- |
| UR-17 | Station availability across a deployment period; no single instrument failure causes total loss of service |
| UR-18 | Station runs for a full unattended deployment period on generated power alone, with no site visit |